/*

Edge-Based Intelligent Traffic Management - ESP32 signal controller

------------------------------------------------------------------

Board   : ESP32 DevKit V1 (ESP32-WROOM-32)

Covers  : Stages 3-10 of the build plan (LEDs, IR sensing, safe 4-approach

state machine, density estimation, fixed vs adaptive control,

emergency priority, sensor fault handling, edge ML hook).

Network : NOT used yet. Telemetry and events are printed as one JSON object

per line on Serial (115200). Stage 11 will publish the same JSON

over MQTT. The lights never depend on the network.

Phase plan: protected 4-phase operation. Exactly one approach (N, S, E or W)

may show green or yellow at a time; all others are red. This makes the

"no conflicting greens" rule trivial to prove and to check at runtime.

Wokwi: each IR sensor is simulated by a slide switch.

Switch LEFT  = sensor clear      (output HIGH)

Switch RIGHT = vehicle detected  (output LOW, same as an FC-51 module)

Serial commands (type in the Serial Monitor, press Enter):

HELP

STATUS

MODE FIXED | MODE ADAPTIVE

EMERGENCY [N|S|E|W]

SET <MIN_GREEN|MAX_GREEN|FIXED_GREEN|YELLOW|ALL_RED|MAX_WAIT|EMERGENCY_GREEN> <ms>

RESET FAULT

RESET STATS

*/

#include <Arduino.h>

#include <ctype.h>

#include <stdlib.h>

#include <string.h>

// ============================ Pin map ======================================

// Avoided on purpose: 0, 2, 5, 12, 15 (boot strapping), 6-11 (flash), 1/3 (USB serial).

const uint8_t NUM_APPROACHES = 4;

const char APPROACH_IDS[NUM_APPROACHES] = {'N', 'S', 'E', 'W'};

const uint8_t RED_PINS[NUM_APPROACHES]    = {4, 18, 22, 26};

const uint8_t YELLOW_PINS[NUM_APPROACHES] = {16, 19, 23, 27};

const uint8_t GREEN_PINS[NUM_APPROACHES]  = {17, 21, 25, 13};

const uint8_t IR_PINS[NUM_APPROACHES]     = {34, 35, 32, 33};  // 34/35 are input-only

const uint8_t EMERGENCY_BUTTON_PIN        = 39;                // input-only, external 10k pull-up

const bool IR_ACTIVE_LOW = true;  // FC-51 / LM393 modules pull LOW when an object is detected

// ============================ Identity =====================================

const char *INTERSECTION_ID = "intersection1";

const char *ML_MODEL_SOURCE = "placeholder-rules";  // becomes "decision-tree-v1" after Stage 9

// ============================ Sensing constants ============================

const uint32_t DEBOUNCE_MS                     = 60;     // raw level must be stable this long

const uint32_t MIN_CLEAR_BEFORE_NEW_VEHICLE_MS = 300;    // stops one car being counted twice

const uint32_t OCCUPANCY_SAMPLE_MS             = 100;

const uint8_t  OCCUPANCY_SAMPLES               = 100;    // 100 x 100 ms = 10 s window

const uint32_t ARRIVAL_WINDOW_MS               = 30000;  // "recent activity" window

const uint8_t  ARRIVAL_HISTORY                 = 16;

const uint32_t STUCK_BLOCKED_MS                = 90000;  // blocked longer than this = suspect sensor

const uint8_t  CHATTER_TRANSITIONS_PER_S       = 12;     // floating/loose wire flips very fast

const uint32_t CHATTER_COOLDOWN_MS             = 5000;

const uint32_t BUTTON_DEBOUNCE_MS              = 50;

// ============================ Control constants ============================

const uint32_t STARTUP_ALL_RED_MS        = 3000;

const uint32_t GAP_OUT_MS                = 3000;  // adaptive: end green early if lane empty this long

const uint32_t GREEN_STEP_PER_LEVEL_MS   = 4000;  // extra green per congestion level

const uint32_t GREEN_BONUS_PER_10S_WAIT  = 1000;

const uint32_t SCORE_PER_CONGESTION_LEVEL = 20;   // adaptive priority score weights

const uint32_t SCORE_PER_WAIT_SECOND      = 1;

const uint32_t TELEMETRY_PERIOD_MS       = 1000;

const uint32_t CONTROL_TICK_MS           = 10;    // loop pacing; all timing still uses millis()

// Hard safety limits. Nothing (serial, MQTT, ML) can configure values outside these.

const uint32_t LIMIT_GREEN_MIN_MS    = 3000;

const uint32_t LIMIT_GREEN_MAX_MS    = 60000;

const uint32_t LIMIT_YELLOW_MIN_MS   = 2000;

const uint32_t LIMIT_YELLOW_MAX_MS   = 6000;

const uint32_t LIMIT_ALL_RED_MIN_MS  = 1000;

const uint32_t LIMIT_ALL_RED_MAX_MS  = 5000;

const uint32_t LIMIT_MAX_WAIT_MIN_MS = 20000;

const uint32_t LIMIT_MAX_WAIT_MAX_MS = 180000;

// ============================ Types ========================================

enum ControlMode : uint8_t { MODE_FIXED, MODE_ADAPTIVE };

enum SignalState : uint8_t { STATE_STARTUP, STATE_GREEN, STATE_YELLOW, STATE_ALL_RED, STATE_FAULT };

enum LightColor : uint8_t { LIGHT_RED, LIGHT_YELLOW, LIGHT_GREEN, LIGHT_OFF };

enum SensorHealth : uint8_t { SENSOR_OK, SENSOR_STUCK, SENSOR_CHATTER };

enum Level : uint8_t { LEVEL_LOW = 0, LEVEL_MEDIUM = 1, LEVEL_HIGH = 2 };

struct TimingConfig {

uint32_t minGreenMs;

uint32_t maxGreenMs;

uint32_t fixedGreenMs;

uint32_t yellowMs;

uint32_t allRedMs;

uint32_t maxWaitMs;        // fairness: a waiting approach is forced next after this

uint32_t emergencyGreenMs;

};

struct ApproachState {

// ---- Measured: comes directly from the IR sensor ----

bool rawBlocked;

bool stableBlocked;

uint32_t rawChangedAtMs;

uint32_t stableChangedAtMs;

uint32_t lastClearedAtMs;

bool occupancy[OCCUPANCY_SAMPLES];

uint8_t occupancyIndex;

uint8_t occupiedCount;

uint32_t arrivalTimesMs[ARRIVAL_HISTORY];

uint8_t arrivalIndex;

uint32_t totalArrivals;

uint8_t transitionsThisSecond;

uint32_t transitionWindowStartMs;

SensorHealth health;

uint32_t healthChangedAtMs;

uint32_t lastValidReadingMs;

// ---- Estimated: derived by rules or the ML model ----

bool hasCall;               // a vehicle has been seen while not green and is not yet served

uint32_t callSinceMs;

Level density;              // rule-based estimate from occupancy + arrivals

Level congestion;           // ML (or placeholder) prediction

uint32_t recommendedGreenMs;

// ---- Statistics for fixed vs adaptive experiments ----

uint32_t totalGreenMs;

uint32_t greenCount;

uint32_t servedCalls;

float sumServedWaitS;

uint32_t maxObservedWaitMs;

};

struct ControllerState {

ControlMode mode;

SignalState state;

int8_t active;              // approach showing green/yellow, -1 when none

int8_t lastServed;

uint32_t stateStartedMs;

uint32_t targetGreenMs;

uint32_t phaseMinGreenMs;   // snapshot so config changes only apply to the next phase

uint32_t phaseDurationMs;   // snapshot of yellow/all-red length

bool emergencyPending;

int8_t emergencyApproach;

bool emergencyPhase;

int8_t defaultEmergencyApproach;

uint32_t unsafeTransitionsBlocked;

uint32_t telemetrySeq;

const char *faultReason;

};

// ============================ Globals ======================================

TimingConfig cfg = {

5000,   // minGreenMs

20000,  // maxGreenMs

10000,  // fixedGreenMs

2000,   // yellowMs

1000,   // allRedMs

45000,  // maxWaitMs

10000   // emergencyGreenMs

};

ApproachState approaches[NUM_APPROACHES];

ControllerState ctl;

LightColor commandedLights[NUM_APPROACHES];

uint32_t lastOccupancySampleMs = 0;

uint32_t lastTelemetryMs = 0;

bool buttonRaw = HIGH;

bool buttonStable = HIGH;

uint32_t buttonChangedAtMs = 0;

char serialLine[96];

uint8_t serialLineLength = 0;

// ============================ Small helpers ================================

const char *levelName(Level level) {

switch (level) {

case LEVEL_LOW: return "LOW";

case LEVEL_MEDIUM: return "MEDIUM";

default: return "HIGH";

}

}

const char *stateName(SignalState state) {

switch (state) {

case STATE_STARTUP: return "STARTUP_ALL_RED";

case STATE_GREEN: return "GREEN";

case STATE_YELLOW: return "YELLOW";

case STATE_ALL_RED: return "ALL_RED";

default: return "FAULT";

}

}

const char *healthName(SensorHealth health) {

switch (health) {

case SENSOR_OK: return "OK";

case SENSOR_STUCK: return "STUCK";

default: return "CHATTER";

}

}

char lightChar(LightColor color) {

switch (color) {

case LIGHT_RED: return 'R';

case LIGHT_YELLOW: return 'Y';

case LIGHT_GREEN: return 'G';

default: return '-';

}

}

const char *modeName(ControlMode mode) {

return mode == MODE_FIXED ? "FIXED" : "ADAPTIVE";

}

char approachChar(int8_t approach) {

return (approach >= 0 && approach < NUM_APPROACHES) ? APPROACH_IDS[approach] : '-';

}

int8_t approachFromChar(char c) {

c = toupper(c);

for (uint8_t i = 0; i < NUM_APPROACHES; i++) {

if (APPROACH_IDS[i] == c) return i;

}

return -1;

}

uint32_t clampU32(uint32_t value, uint32_t low, uint32_t high) {

if (value < low) return low;

if (value > high) return high;

return value;

}

uint32_t waitingMs(const ApproachState &a, uint32_t now) {

return a.hasCall ? now - a.callSinceMs : 0;

}

void printEventHeader(const char *eventName, uint32_t now) {

Serial.printf("{\"type\":\"event\",\"intersection_id\":\"%s\",\"uptime_ms\":%lu,\"event\":\"%s\"",

INTERSECTION_ID, (unsigned long)now, eventName);

}

// ============================ Config validation ============================

bool validateConfig(const TimingConfig &c, const char **error) {

if (c.minGreenMs < LIMIT_GREEN_MIN_MS || c.minGreenMs > LIMIT_GREEN_MAX_MS) { *error = "min_green out of limits"; return false; }

if (c.maxGreenMs < c.minGreenMs || c.maxGreenMs > LIMIT_GREEN_MAX_MS)     { *error = "max_green must be >= min_green and <= limit"; return false; }

if (c.fixedGreenMs < c.minGreenMs || c.fixedGreenMs > c.maxGreenMs)       { *error = "fixed_green must be between min_green and max_green"; return false; }

if (c.yellowMs < LIMIT_YELLOW_MIN_MS || c.yellowMs > LIMIT_YELLOW_MAX_MS)  { *error = "yellow out of limits"; return false; }

if (c.allRedMs < LIMIT_ALL_RED_MIN_MS || c.allRedMs > LIMIT_ALL_RED_MAX_MS) { *error = "all_red out of limits"; return false; }

if (c.maxWaitMs < LIMIT_MAX_WAIT_MIN_MS || c.maxWaitMs > LIMIT_MAX_WAIT_MAX_MS) { *error = "max_wait out of limits"; return false; }

if (c.emergencyGreenMs < c.minGreenMs || c.emergencyGreenMs > LIMIT_GREEN_MAX_MS) { *error = "emergency_green out of limits"; return false; }

*error = "";

return true;

}

// ============================ Sensing ======================================

void setSensorHealth(uint8_t i, SensorHealth health, uint32_t now) {

ApproachState &a = approaches[i];

if (a.health == health) return;

a.health = health;

a.healthChangedAtMs = now;

printEventHeader("sensor_health", now);

Serial.printf(",\"approach\":\"%c\",\"health\":\"%s\"}\n", APPROACH_IDS[i], healthName(health));

}

void recordArrival(ApproachState &a, uint32_t now) {

a.arrivalTimesMs[a.arrivalIndex] = now;

a.arrivalIndex = (a.arrivalIndex + 1) % ARRIVAL_HISTORY;

a.totalArrivals++;

}

uint8_t countRecentArrivals(const ApproachState &a, uint32_t now) {

uint8_t count = 0;

for (uint8_t k = 0; k < ARRIVAL_HISTORY; k++) {

uint32_t t = a.arrivalTimesMs[k];

if (t != 0 && now - t <= ARRIVAL_WINDOW_MS) count++;

}

return count;

}

void updateSensors(uint32_t now) {

for (uint8_t i = 0; i < NUM_APPROACHES; i++) {

ApproachState &a = approaches[i];

int level = digitalRead(IR_PINS[i]);

bool blocked = IR_ACTIVE_LOW ? (level == LOW) : (level == HIGH);

if (blocked != a.rawBlocked) {

a.rawBlocked = blocked;

a.rawChangedAtMs = now;

if (a.transitionsThisSecond < 255) a.transitionsThisSecond++;

}

// Chatter check once per second (a floating or loose wire toggles very fast).

if (now - a.transitionWindowStartMs >= 1000) {

if (a.transitionsThisSecond > CHATTER_TRANSITIONS_PER_S) {

setSensorHealth(i, SENSOR_CHATTER, now);

a.healthChangedAtMs = now;  // keep extending the cooldown while chattering

} else if (a.health == SENSOR_CHATTER && now - a.healthChangedAtMs >= CHATTER_COOLDOWN_MS) {

setSensorHealth(i, SENSOR_OK, now);

}

a.transitionsThisSecond = 0;

a.transitionWindowStartMs = now;

}

// Debounce: accept a new level only after it has been stable for DEBOUNCE_MS.

if (a.rawBlocked != a.stableBlocked && now - a.rawChangedAtMs >= DEBOUNCE_MS) {

a.stableBlocked = a.rawBlocked;

a.stableChangedAtMs = now;

if (a.stableBlocked) {

if (now - a.lastClearedAtMs >= MIN_CLEAR_BEFORE_NEW_VEHICLE_MS) recordArrival(a, now);

} else {

a.lastClearedAtMs = now;

}

}

// Stuck check: permanently blocked usually means a fault or a parked object.

if (a.stableBlocked && now - a.stableChangedAtMs >= STUCK_BLOCKED_MS) {

setSensorHealth(i, SENSOR_STUCK, now);

} else if (a.health == SENSOR_STUCK && !a.stableBlocked) {

setSensorHealth(i, SENSOR_OK, now);

}

if (a.health == SENSOR_OK) a.lastValidReadingMs = now;

}

if (now - lastOccupancySampleMs >= OCCUPANCY_SAMPLE_MS) {

lastOccupancySampleMs = now;

for (uint8_t i = 0; i < NUM_APPROACHES; i++) {

ApproachState &a = approaches[i];

if (a.occupancy[a.occupancyIndex]) a.occupiedCount--;

a.occupancy[a.occupancyIndex] = a.stableBlocked;

if (a.stableBlocked) a.occupiedCount++;

a.occupancyIndex = (a.occupancyIndex + 1) % OCCUPANCY_SAMPLES;

}

}

}

uint8_t occupancyPercent(const ApproachState &a) {

return (uint8_t)((uint16_t)a.occupiedCount * 100 / OCCUPANCY_SAMPLES);

}

// ============================ ML hook ======================================

// Feature vector order. Stage 9 must train with exactly this order and save it

// in ml/feature_config.json:

//   0 density_level (0..2)   1 occupancy_pct (0..100)   2 arrivals_30s

//   3 wait_s                 4 was_last_served (0/1)

const uint8_t NUM_FEATURES = 5;

// PLACEHOLDER rules with the same inputs/outputs as the future Decision Tree.

// Replace the body with the micromlgen-generated predict() after Stage 9.

Level predictCongestion(const float *x) {

float density = x[0], occupancy = x[1], arrivals = x[2], waitS = x[3];

if (waitS >= 30.0f && density >= 1.0f) return LEVEL_HIGH;

if (density >= 2.0f) return (occupancy >= 80.0f || arrivals >= 4.0f) ? LEVEL_HIGH : LEVEL_MEDIUM;

if (density >= 1.0f) return waitS >= 15.0f ? LEVEL_HIGH : LEVEL_MEDIUM;

return arrivals >= 2.0f ? LEVEL_MEDIUM : LEVEL_LOW;

}

uint32_t recommendGreenMs(Level congestion, uint32_t waitMsValue) {

uint32_t bonusSteps = waitMsValue / 10000;

if (bonusSteps > 3) bonusSteps = 3;

uint32_t green = cfg.minGreenMs + (uint32_t)congestion * GREEN_STEP_PER_LEVEL_MS + bonusSteps * GREEN_BONUS_PER_10S_WAIT;

return clampU32(green, cfg.minGreenMs, cfg.maxGreenMs);

}

// ============================ Estimation ===================================

void updateEstimates(uint32_t now) {

for (uint8_t i = 0; i < NUM_APPROACHES; i++) {

ApproachState &a = approaches[i];

uint8_t occupancy = occupancyPercent(a);

uint8_t arrivals = countRecentArrivals(a, now);

// Rule-based density estimate. A faulty sensor falls back to MEDIUM so the

// approach is neither starved nor allowed to dominate.

if (a.health != SENSOR_OK) {

a.density = LEVEL_MEDIUM;

} else if (occupancy >= 60 || arrivals >= 6) {

a.density = LEVEL_HIGH;

} else if (occupancy >= 25 || arrivals >= 3) {

a.density = LEVEL_MEDIUM;

} else {

a.density = LEVEL_LOW;

}

// Latch a "call" when a vehicle is seen while this approach is not green.

bool servingNow = (ctl.state == STATE_GREEN && ctl.active == i);

bool demand = a.stableBlocked || a.health != SENSOR_OK;

if (!servingNow && demand && !a.hasCall) {

a.hasCall = true;

a.callSinceMs = now;

}

uint32_t waitNow = waitingMs(a, now);

if (waitNow > a.maxObservedWaitMs) a.maxObservedWaitMs = waitNow;

float features[NUM_FEATURES] = {

(float)a.density,

(float)occupancy,

(float)arrivals,

waitNow / 1000.0f,

(ctl.lastServed == i) ? 1.0f : 0.0f

};

a.congestion = predictCongestion(features);

a.recommendedGreenMs = recommendGreenMs(a.congestion, waitNow);

}

}

// ============================ Controller ===================================

bool allSensorsFaulty() {

for (uint8_t i = 0; i < NUM_APPROACHES; i++) {

if (approaches[i].health == SENSOR_OK) return false;

}

return true;

}

ControlMode effectiveMode() {

// Adaptive control is meaningless without any working sensor.

if (ctl.mode == MODE_ADAPTIVE && allSensorsFaulty()) return MODE_FIXED;

return ctl.mode;

}

bool anyOtherCall(int8_t except) {

for (uint8_t i = 0; i < NUM_APPROACHES; i++) {

if (i != except && approaches[i].hasCall) return true;

}

return false;

}

void enterState(SignalState state, uint32_t now) {

ctl.state = state;

ctl.stateStartedMs = now;

printEventHeader("phase_change", now);

Serial.printf(",\"state\":\"%s\",\"approach\":\"%c\"}\n", stateName(state), approachChar(ctl.active));

}

void enterFault(const char *reason, uint32_t now) {

ctl.faultReason = reason;

ctl.active = -1;

ctl.emergencyPhase = false;

enterState(STATE_FAULT, now);

printEventHeader("controller_fault", now);

Serial.printf(",\"reason\":\"%s\"}\n", reason);

}

void chooseAdaptive(uint32_t now, int8_t &next, uint32_t &greenMs, const char *&reason) {

// 1. Fairness guard: anyone waiting longer than maxWait goes first (longest wait wins).

int8_t best = -1;

uint32_t bestWait = 0;

for (uint8_t i = 0; i < NUM_APPROACHES; i++) {

uint32_t w = waitingMs(approaches[i], now);

if (approaches[i].hasCall && w >= cfg.maxWaitMs && w > bestWait) {

best = i;

bestWait = w;

}

}

if (best >= 0) {

next = best;

greenMs = approaches[best].recommendedGreenMs;

reason = "max_wait_fairness";

return;

}

// 2. Priority score = ML congestion level + waiting time. The approach that

//    was just served is skipped while anyone else is waiting.

bool othersWaiting = anyOtherCall(ctl.lastServed);

uint32_t bestScore = 0;

for (uint8_t i = 0; i < NUM_APPROACHES; i++) {

const ApproachState &a = approaches[i];

if (!a.hasCall) continue;

if (i == ctl.lastServed && othersWaiting) continue;

uint32_t score = (uint32_t)a.congestion * SCORE_PER_CONGESTION_LEVEL +

(waitingMs(a, now) / 1000) * SCORE_PER_WAIT_SECOND + 1;

if (score > bestScore) {

bestScore = score;

best = i;

}

}

if (best >= 0) {

next = best;

greenMs = approaches[best].recommendedGreenMs;

reason = "adaptive_score";

return;

}

// 3. No demand anywhere: keep rotating with minimum green.

next = (ctl.lastServed + 1) % NUM_APPROACHES;

greenMs = cfg.minGreenMs;

reason = "no_demand_rotation";

}

void startNextGreen(uint32_t now) {

int8_t next = -1;

uint32_t greenMs = cfg.minGreenMs;

const char *reason = "";

if (ctl.emergencyPending) {

next = ctl.emergencyApproach;

greenMs = cfg.emergencyGreenMs;

reason = "emergency";

ctl.emergencyPending = false;

ctl.emergencyPhase = true;

} else if (effectiveMode() == MODE_FIXED) {

next = (ctl.lastServed + 1) % NUM_APPROACHES;

greenMs = cfg.fixedGreenMs;

reason = "fixed_rotation";

} else {

chooseAdaptive(now, next, greenMs, reason);

}

// Local validation of every recommendation before it touches the lights.

if (next < 0 || next >= NUM_APPROACHES) {

enterFault("invalid_next_approach", now);

return;

}

uint32_t upper = ctl.emergencyPhase ? LIMIT_GREEN_MAX_MS : cfg.maxGreenMs;

greenMs = clampU32(greenMs, cfg.minGreenMs, upper);

ApproachState &a = approaches[next];

uint32_t servedWait = waitingMs(a, now);

if (a.hasCall) {

a.sumServedWaitS += servedWait / 1000.0f;

a.servedCalls++;

a.hasCall = false;

}

a.greenCount++;

ctl.active = next;

ctl.targetGreenMs = greenMs;

ctl.phaseMinGreenMs = cfg.minGreenMs;

printEventHeader("green_decision", now);

Serial.printf(",\"approach\":\"%c\",\"green_ms\":%lu,\"reason\":\"%s\",\"mode\":\"%s\",\"congestion\":\"%s\",\"density\":\"%s\",\"served_wait_s\":%.1f}\n",

APPROACH_IDS[next], (unsigned long)greenMs, reason, modeName(effectiveMode()),

levelName(a.congestion), levelName(a.density), servedWait / 1000.0f);

enterState(STATE_GREEN, now);

}

bool shouldEndGreen(uint32_t now, uint32_t elapsed) {

// Emergency for the approach that is already green: extend instead of switching.

if (ctl.emergencyPending && ctl.emergencyApproach == ctl.active) {

uint32_t extended = elapsed + cfg.emergencyGreenMs;

if (extended > ctl.targetGreenMs) ctl.targetGreenMs = clampU32(extended, cfg.minGreenMs, LIMIT_GREEN_MAX_MS);

ctl.emergencyPending = false;

ctl.emergencyPhase = true;

printEventHeader("emergency_extend", now);

Serial.printf(",\"approach\":\"%c\",\"green_ms\":%lu}\n", approachChar(ctl.active), (unsigned long)ctl.targetGreenMs);

return false;

}

if (elapsed < ctl.phaseMinGreenMs) return false;  // never cut below minimum green

if (elapsed >= ctl.targetGreenMs) return true;

if (ctl.emergencyPending) return true;            // emergency for another approach

if (effectiveMode() == MODE_ADAPTIVE && !ctl.emergencyPhase) {

const ApproachState &a = approaches[ctl.active];

bool laneEmpty = a.health == SENSOR_OK && !a.stableBlocked && now - a.stableChangedAtMs >= GAP_OUT_MS;

if (laneEmpty && anyOtherCall(ctl.active)) {

printEventHeader("gap_out", now);

Serial.printf(",\"approach\":\"%c\",\"green_used_ms\":%lu}\n", approachChar(ctl.active), (unsigned long)elapsed);

return true;

}

}

return false;

}

void updateController(uint32_t now) {

uint32_t elapsed = now - ctl.stateStartedMs;

switch (ctl.state) {

case STATE_STARTUP:

if (elapsed >= STARTUP_ALL_RED_MS) startNextGreen(now);

break;

case STATE_GREEN:

if (shouldEndGreen(now, elapsed)) {

approaches[ctl.active].totalGreenMs += elapsed;

ctl.phaseDurationMs = cfg.yellowMs;

enterState(STATE_YELLOW, now);

}

break;

case STATE_YELLOW:

if (elapsed >= ctl.phaseDurationMs) {

ctl.lastServed = ctl.active;

ctl.active = -1;

ctl.emergencyPhase = false;

ctl.phaseDurationMs = cfg.allRedMs;

enterState(STATE_ALL_RED, now);

}

break;

case STATE_ALL_RED:

if (elapsed >= ctl.phaseDurationMs) startNextGreen(now);

break;

case STATE_FAULT:

break;  // flashing red until "RESET FAULT"

}

}

// ============================ Outputs ======================================

void applyOutputs(uint32_t now) {

LightColor desired[NUM_APPROACHES];

for (uint8_t i = 0; i < NUM_APPROACHES; i++) desired[i] = LIGHT_RED;

if (ctl.state == STATE_GREEN && ctl.active >= 0) desired[ctl.active] = LIGHT_GREEN;

if (ctl.state == STATE_YELLOW && ctl.active >= 0) desired[ctl.active] = LIGHT_YELLOW;

if (ctl.state == STATE_FAULT) {

LightColor flash = ((now / 500) % 2 == 0) ? LIGHT_RED : LIGHT_OFF;

for (uint8_t i = 0; i < NUM_APPROACHES; i++) desired[i] = flash;

}

// Independent safety check: at most one approach may be green or yellow.

uint8_t moving = 0;

for (uint8_t i = 0; i < NUM_APPROACHES; i++) {

if (desired[i] == LIGHT_GREEN || desired[i] == LIGHT_YELLOW) moving++;

}

if (moving > 1) {

ctl.unsafeTransitionsBlocked++;

for (uint8_t i = 0; i < NUM_APPROACHES; i++) desired[i] = LIGHT_RED;

enterFault("conflicting_green_blocked", now);

}

bool changed = false;

for (uint8_t i = 0; i < NUM_APPROACHES; i++) {

if (desired[i] != commandedLights[i]) changed = true;

}

if (!changed) return;  // only touch the pins when a light actually changes

// Turn everything that must go OFF first, then turn ON, so two greens can

// never overlap even for one instruction.

for (uint8_t i = 0; i < NUM_APPROACHES; i++) {

if (desired[i] != LIGHT_GREEN) digitalWrite(GREEN_PINS[i], LOW);

if (desired[i] != LIGHT_YELLOW) digitalWrite(YELLOW_PINS[i], LOW);

if (desired[i] != LIGHT_RED) digitalWrite(RED_PINS[i], LOW);

}

for (uint8_t i = 0; i < NUM_APPROACHES; i++) {

if (desired[i] == LIGHT_GREEN) digitalWrite(GREEN_PINS[i], HIGH);

if (desired[i] == LIGHT_YELLOW) digitalWrite(YELLOW_PINS[i], HIGH);

if (desired[i] == LIGHT_RED) digitalWrite(RED_PINS[i], HIGH);

commandedLights[i] = desired[i];

}

}

// ============================ Emergency ====================================

void requestEmergency(int8_t approach, const char *source, uint32_t now) {

if (ctl.state == STATE_FAULT || ctl.emergencyPending) {

printEventHeader("emergency_ignored", now);

Serial.printf(",\"source\":\"%s\",\"reason\":\"%s\"}\n", source,

ctl.state == STATE_FAULT ? "controller_fault" : "already_pending");

return;

}

ctl.emergencyPending = true;

ctl.emergencyApproach = approach;

printEventHeader("emergency_request", now);

Serial.printf(",\"approach\":\"%c\",\"source\":\"%s\"}\n", approachChar(approach), source);

}

void updateEmergencyButton(uint32_t now) {

bool level = digitalRead(EMERGENCY_BUTTON_PIN);

if (level != buttonRaw) {

buttonRaw = level;

buttonChangedAtMs = now;

}

if (buttonRaw != buttonStable && now - buttonChangedAtMs >= BUTTON_DEBOUNCE_MS) {

buttonStable = buttonRaw;

if (buttonStable == LOW) requestEmergency(ctl.defaultEmergencyApproach, "button", now);

}

}

// ============================ Telemetry ====================================

void publishTelemetry(uint32_t now) {

if (now - lastTelemetryMs < TELEMETRY_PERIOD_MS) return;

lastTelemetryMs = now;

ctl.telemetrySeq++;

Serial.printf("{\"type\":\"telemetry\",\"intersection_id\":\"%s\",\"seq\":%lu,\"uptime_ms\":%lu,",

INTERSECTION_ID, (unsigned long)ctl.telemetrySeq, (unsigned long)now);

Serial.printf("\"mode\":\"%s\",\"effective_mode\":\"%s\",\"state\":\"%s\",\"active\":\"%c\",",

modeName(ctl.mode), modeName(effectiveMode()), stateName(ctl.state), approachChar(ctl.active));

Serial.printf("\"phase_elapsed_ms\":%lu,\"target_green_ms\":%lu,\"emergency_pending\":%s,\"emergency_phase\":%s,",

(unsigned long)(now - ctl.stateStartedMs), (unsigned long)ctl.targetGreenMs,

ctl.emergencyPending ? "true" : "false", ctl.emergencyPhase ? "true" : "false");

Serial.printf("\"inference\":\"edge\",\"ml_source\":\"%s\",\"unsafe_transitions_blocked\":%lu,\"approaches\":[",

ML_MODEL_SOURCE, (unsigned long)ctl.unsafeTransitionsBlocked);

for (uint8_t i = 0; i < NUM_APPROACHES; i++) {

const ApproachState &a = approaches[i];

Serial.printf("%s{\"id\":\"%c\",\"light\":\"%c\",\"health\":\"%s\",\"last_valid_ms\":%lu,",

i == 0 ? "" : ",", APPROACH_IDS[i], lightChar(commandedLights[i]), healthName(a.health),

(unsigned long)a.lastValidReadingMs);

Serial.printf("\"measured\":{\"blocked\":%s,\"occupancy_pct\":%u,\"arrivals_30s\":%u,\"total_arrivals\":%lu},",

a.stableBlocked ? "true" : "false", occupancyPercent(a), countRecentArrivals(a, now),

(unsigned long)a.totalArrivals);

Serial.printf("\"estimated\":{\"call\":%s,\"wait_s\":%.1f,\"density\":\"%s\",\"congestion\":\"%s\",\"recommended_green_ms\":%lu}}",

a.hasCall ? "true" : "false", waitingMs(a, now) / 1000.0f, levelName(a.density),

levelName(a.congestion), (unsigned long)a.recommendedGreenMs);

}

Serial.print("]}\n");

}

void printStatus(uint32_t now) {

Serial.printf("{\"type\":\"status\",\"uptime_ms\":%lu,\"mode\":\"%s\",\"state\":\"%s\",\"fault_reason\":\"%s\",",

(unsigned long)now, modeName(ctl.mode), stateName(ctl.state), ctl.faultReason);

Serial.printf("\"config\":{\"min_green_ms\":%lu,\"max_green_ms\":%lu,\"fixed_green_ms\":%lu,\"yellow_ms\":%lu,\"all_red_ms\":%lu,\"max_wait_ms\":%lu,\"emergency_green_ms\":%lu},\"stats\":[",

(unsigned long)cfg.minGreenMs, (unsigned long)cfg.maxGreenMs, (unsigned long)cfg.fixedGreenMs,

(unsigned long)cfg.yellowMs, (unsigned long)cfg.allRedMs, (unsigned long)cfg.maxWaitMs,

(unsigned long)cfg.emergencyGreenMs);

for (uint8_t i = 0; i < NUM_APPROACHES; i++) {

const ApproachState &a = approaches[i];

float avgWait = a.servedCalls ? a.sumServedWaitS / a.servedCalls : 0.0f;

Serial.printf("%s{\"id\":\"%c\",\"green_time_s\":%.1f,\"green_count\":%lu,\"served_calls\":%lu,\"avg_wait_s\":%.1f,\"max_wait_s\":%.1f,\"total_arrivals\":%lu}",

i == 0 ? "" : ",", APPROACH_IDS[i], a.totalGreenMs / 1000.0f, (unsigned long)a.greenCount,

(unsigned long)a.servedCalls, avgWait, a.maxObservedWaitMs / 1000.0f, (unsigned long)a.totalArrivals);

}

Serial.print("]}\n");

}

void resetStats() {

for (uint8_t i = 0; i < NUM_APPROACHES; i++) {

ApproachState &a = approaches[i];

a.totalGreenMs = 0;

a.greenCount = 0;

a.servedCalls = 0;

a.sumServedWaitS = 0;

a.maxObservedWaitMs = 0;

a.totalArrivals = 0;

}

ctl.unsafeTransitionsBlocked = 0;

}

// ============================ Serial commands ==============================

void printHelp() {

Serial.println("Commands: HELP | STATUS | MODE FIXED | MODE ADAPTIVE | EMERGENCY [N|S|E|W]");

Serial.println("          SET <MIN_GREEN|MAX_GREEN|FIXED_GREEN|YELLOW|ALL_RED|MAX_WAIT|EMERGENCY_GREEN> <ms>");

Serial.println("          RESET FAULT | RESET STATS");

}

void commandResult(const char *command, bool ok, const char *message, uint32_t now) {

printEventHeader("command_result", now);

Serial.printf(",\"command\":\"%s\",\"ok\":%s,\"message\":\"%s\"}\n", command, ok ? "true" : "false", message);

}

void handleCommand(char *line, uint32_t now) {

for (char *p = line; *p; p++) *p = toupper(*p);

if (strcmp(line, "HELP") == 0) {

printHelp();

} else if (strcmp(line, "STATUS") == 0) {

printStatus(now);

} else if (strcmp(line, "MODE FIXED") == 0 || strcmp(line, "MODE ADAPTIVE") == 0) {

ctl.mode = (line[5] == 'F') ? MODE_FIXED : MODE_ADAPTIVE;

commandResult("MODE", true, "applies from the next green decision", now);

} else if (strncmp(line, "EMERGENCY", 9) == 0) {

int8_t approach = ctl.defaultEmergencyApproach;

if (strlen(line) > 10) approach = approachFromChar(line[10]);

if (approach < 0) {

commandResult("EMERGENCY", false, "approach must be N, S, E or W", now);

} else {

requestEmergency(approach, "serial", now);

}

} else if (strncmp(line, "SET ", 4) == 0) {

char *name = line + 4;

char *valueText = strchr(name, ' ');

if (!valueText) {

commandResult("SET", false, "usage: SET <NAME> <ms>", now);

return;

}

*valueText++ = '\0';

char *end;

unsigned long value = strtoul(valueText, &end, 10);

if (end == valueText) {

commandResult("SET", false, "value must be a number of milliseconds", now);

return;

}

TimingConfig candidate = cfg;

if (strcmp(name, "MIN_GREEN") == 0) candidate.minGreenMs = value;

else if (strcmp(name, "MAX_GREEN") == 0) candidate.maxGreenMs = value;

else if (strcmp(name, "FIXED_GREEN") == 0) candidate.fixedGreenMs = value;

else if (strcmp(name, "YELLOW") == 0) candidate.yellowMs = value;

else if (strcmp(name, "ALL_RED") == 0) candidate.allRedMs = value;

else if (strcmp(name, "MAX_WAIT") == 0) candidate.maxWaitMs = value;

else if (strcmp(name, "EMERGENCY_GREEN") == 0) candidate.emergencyGreenMs = value;

else {

commandResult("SET", false, "unknown parameter", now);

return;

}

const char *error;

if (validateConfig(candidate, &error)) {

cfg = candidate;

commandResult("SET", true, "applies from the next phase", now);

} else {

commandResult("SET", false, error, now);

}

} else if (strcmp(line, "RESET FAULT") == 0) {

if (ctl.state == STATE_FAULT) {

ctl.faultReason = "";

ctl.active = -1;

enterState(STATE_STARTUP, now);

commandResult("RESET FAULT", true, "restarting with all-red", now);

} else {

commandResult("RESET FAULT", false, "controller is not in fault", now);

}

} else if (strcmp(line, "RESET STATS") == 0) {

resetStats();

commandResult("RESET STATS", true, "statistics cleared", now);

} else if (line[0] != '\0') {

commandResult("UNKNOWN", false, "type HELP", now);

}

}

void handleSerialCommands(uint32_t now) {

while (Serial.available() > 0) {

char c = (char)Serial.read();

if (c == '\r') continue;

if (c == '\n') {

serialLine[serialLineLength] = '\0';

handleCommand(serialLine, now);

serialLineLength = 0;

} else if (serialLineLength < sizeof(serialLine) - 1) {

serialLine[serialLineLength++] = c;

}

}

}

// ============================ Setup / loop =================================

void setup() {

Serial.begin(115200);

// Outputs first, all red: the intersection is safe from the first millisecond.

for (uint8_t i = 0; i < NUM_APPROACHES; i++) {

pinMode(RED_PINS[i], OUTPUT);

pinMode(YELLOW_PINS[i], OUTPUT);

pinMode(GREEN_PINS[i], OUTPUT);

digitalWrite(GREEN_PINS[i], LOW);

digitalWrite(YELLOW_PINS[i], LOW);

digitalWrite(RED_PINS[i], HIGH);

commandedLights[i] = LIGHT_RED;

pinMode(IR_PINS[i], INPUT);  // the IR module drives the line; 34/35 have no internal pull-ups anyway

}

pinMode(EMERGENCY_BUTTON_PIN, INPUT);  // external 10k pull-up to 3V3

uint32_t now = millis();

memset(approaches, 0, sizeof(approaches));

for (uint8_t i = 0; i < NUM_APPROACHES; i++) {

ApproachState &a = approaches[i];

int level = digitalRead(IR_PINS[i]);

a.rawBlocked = IR_ACTIVE_LOW ? (level == LOW) : (level == HIGH);

a.stableBlocked = a.rawBlocked;

a.rawChangedAtMs = now;

a.stableChangedAtMs = now;

a.transitionWindowStartMs = now;

a.health = SENSOR_OK;

a.density = LEVEL_LOW;

a.congestion = LEVEL_LOW;

a.recommendedGreenMs = cfg.minGreenMs;

}

buttonRaw = buttonStable = digitalRead(EMERGENCY_BUTTON_PIN);

memset(&ctl, 0, sizeof(ctl));

ctl.mode = MODE_ADAPTIVE;

ctl.active = -1;

ctl.lastServed = NUM_APPROACHES - 1;  // so the first fixed-time green is North

ctl.emergencyApproach = -1;

ctl.defaultEmergencyApproach = 0;    // the button requests North

ctl.faultReason = "";

const char *error;

if (!validateConfig(cfg, &error)) {

Serial.printf("{\"type\":\"event\",\"event\":\"bad_default_config\",\"message\":\"%s\"}\n", error);

enterFault("bad_default_config", now);

return;

}

Serial.println();

Serial.println("Smart traffic controller ready. Lights run locally; no network needed.");

printHelp();

enterState(STATE_STARTUP, now);

}

void loop() {

uint32_t now = millis();

handleSerialCommands(now);

updateSensors(now);

updateEmergencyButton(now);

updateEstimates(now);

updateController(now);

applyOutputs(now);

publishTelemetry(now);

delay(CONTROL_TICK_MS);  // 10 ms tick: far below every debounce/phase time, frees the CPU

}